# Dionisi preset 23 – St Triad detune

## Sequencing

Melody into V/Oct or MIDI (Channel 1) input of Dionisi.

No other input required.

Master output: Mix 1 & 2.

## Controls

Make a chord with the first 3 CV’s, stereo detune per note of the triad via CV 4 through 6

CV1 = Equal temp. quantized note (1 oct range) for VCO’s 1 & 4

CV2 = Equal temp. quantized note (1 oct range) for VCO’s 2 & 5

CV3 = Equal temp. quantized note (1 oct range) for VCO’s 3 & 6

CV4 = Detune up VCO 1, detune down VCO 4

CV5 = Detune down VCO 2, detune up VCO 5

CV6 = Detune up VCO 3, detune down VCO 6

## Outputs & comments

Mix 1 = VCO 1 + VCO 2 + VCO 3

Mix 2 = VCO 4 + VCO 5 + VCO6

Saw wave based

| ## Oscillator | O1 | O2 | O3 | O4 | O5 | O6 |
| --- | --- | --- | --- | --- | --- | --- |
| Range | Audio | Audio | Audio | Audio | Audio | Audio |
| Pitch Mode | Ratio | Ratio | Ratio | Ratio | Ratio | Ratio |
| Ratio | 1.000x | 1.000x | 1.000x | 1.000x | 1.000x | 1.000x |
| Wave | Saw | Saw | Saw | Saw | Saw | Saw |
| Quantise | Off | Off | Off | Off | Off | Off |
| Sync Input | Off | Off | Off | Off | Off | Off |
| Sync State | Pos e. | Pos e. | Pos e. | Pos e. | Pos e. | Pos e. |
| Midi Chan. | 1 | 1 | 1 | 1 | 1 | 1 |
| PB Range | 0 | 0 | 0 | 0 | 0 | 0 |

| ## Detune | O1 | O2 | O3 | O4 | O5 | O6 |
| --- | --- | --- | --- | --- | --- | --- |
| Detune Quantise | Off | Off | Off | Off | Off | Off |
| Detune Amount | 0ct | 0ct | 0ct | 0ct | 0ct | 0ct |
| Fine Hz Amount | 0.0Hz | 0.0Hz | 0.0Hz | 0.0Hz | 0.0Hz | 0.0Hz |

| ## Exp FM | O1 | O2 | O3 | O4 | O5 | O6 |
| --- | --- | --- | --- | --- | --- | --- |
| Input | CV1 | CV2 | CV3 | CV1 | CV2 | CV3 |
| Depth | 0.100 | 0.100 | 0.100 | 0.100 | 0.100 | 0.100 |
| Idx Inp. | Off | Off | Off | Off | Off | Off |
| Idx Max. | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| Idx Min. | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |

| ## Lin FM | O1 | O2 | O3 | O4 | O5 | O6 |
| --- | --- | --- | --- | --- | --- | --- |
| Input | CV4 | CV5 | CV6 | CV4 | CV5 | CV6 |
| Depth | 0.002 | -0.002 | 0.002 | -0.002 | 0.002 | -0.002 |
| Idx Inp. | Off | Off | Off | Off | Off | Off |
| Idx Max. | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| Idx Min. | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |

| ## Phase | O1 | O2 | O3 | O4 | O5 | O6 |
| --- | --- | --- | --- | --- | --- | --- |
| Initial | 0dg | 0dg | 0dg | 0dg | 0dg | 0dg |
| Input | Off | Off | Off | Off | Off | Off |
| Depth | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| Idx Inp. | Off | Off | Off | Off | Off | Off |
| Idx Max. | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| Idx Min. | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |

| ## AM | O1 | O2 | O3 | O4 | O5 | O6 |
| --- | --- | --- | --- | --- | --- | --- |
| Input | Off | Off | Off | Off | Off | Off |
| Depth | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| Bias | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| Idx Inp. | Off | Off | Off | Off | Off | Off |
| Idx Max. | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| Idx Min. | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |

| ## M1 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Input | VCO1 | VCO2 | VCO3 | Off | Off | Off |
| Level | 100 | 100 | 100 | 0 | 0 | 0 |

Clipping		Hard

Limiting		Min.

| ## M2 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Input | VCO4 | VCO5 | VCO6 | Off | Off | Off |
| Level | 100 | 100 | 100 | 0 | 0 | 0 |

Clipping		Hard

Limiting		Min.