# Dionisi preset 24 – St MajChd Tri

## Sequencing

Melody into V/Oct or MIDI (Channel 1) input of Dionisi.

No other input required.

Master output: Mix 1 & 2.

## Controls

Major chord with a pair of VCO’s per note of the triad, stereo detune spread via CV 6

PM and AM controls via CV 1-5

CV1 = PM VCO 1 & 4 with VCO 5 & 2 (Chord note 1/3 and 2/3)

CV2 = PM VCO 2 & 5 with VCO 6 & 3 (Chord note 2/3 and 3/3)

CV3 = PM VCO 3 & 6 with VCO 4 & 1 (Chord note 3/3 and 1/3) = x-modulation (very attenuated)

CV4 = AM VCO 1 & 4 with VCO 3 & 6 (Chord note 1/3 and 3/3)

CV5 = AM VCO 2 & 5 with VCO 1 &4 (Chord note 2/3 and 1/3)

CV6 = Detune up VCO 1 3 5, detune down VCO 2 4 6

## Outputs & comments

Mix 1 = VCO 1 + VCO 2 + VCO 3

Mix 2 = VCO 4 + VCO 5 + VCO6

Triangle wave based

| ## Oscillator | O1 | O2 | O3 | O4 | O5 | O6 |
| --- | --- | --- | --- | --- | --- | --- |
| Range | Audio | Audio | Audio | Audio | Audio | Audio |
| Pitch Mode | Ratio | Ratio | Ratio | Ratio | Ratio | Ratio |
| Ratio | 1.000x | 1.000x | 1.000x | 1.000x | 1.000x | 1.000x |
| Wave | Tri | Tri | Tri | Tri | Tri | Tri |
| Quantise | Off | Off | Off | Off | Off | Off |
| Sync Input | Off | Off | Off | Off | Off | Off |
| Sync State | Pos e. | Pos e. | Pos e. | Pos e. | Pos e. | Pos e. |
| Midi Chan. | 1 | 1 | 1 | 1 | 1 | 1 |
| PB Range | 0 | 0 | 0 | 0 | 0 | 0 |

| ## Detune | O1 | O2 | O3 | O4 | O5 | O6 |
| --- | --- | --- | --- | --- | --- | --- |
| Detune Quantise | Equal | Equal | Equal | Off | Equal | Equal |
| Detune Amount | 4ST | 4ST | 7ST | 0ct | 4ST | 7ST |
| Fine Hz Amount | 0.0Hz | 0.0Hz | 0.0Hz | 0.0Hz | 0.0Hz | 0.0Hz |

| ## Exp FM | O1 | O2 | O3 | O4 | O5 | O6 |
| --- | --- | --- | --- | --- | --- | --- |
| Input | Off | Off | Off | Off | Off | Off |
| Depth | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| Idx Inp. | Off | Off | Off | Off | Off | Off |
| Idx Max. | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| Idx Min. | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |

| ## Lin FM | O1 | O2 | O3 | O4 | O5 | O6 |
| --- | --- | --- | --- | --- | --- | --- |
| Input | CV6 | CV6 | CV6 | CV6 | CV6 | CV6 |
| Depth | 0.001 | -0.002 | 0.003 | -0.001 | 0.002 | -0.003 |
| Idx Inp. | Off | Off | Off | Off | Off | Off |
| Idx Max. | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| Idx Min. | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |

| ## Phase | O1 | O2 | O3 | O4 | O5 | O6 |
| --- | --- | --- | --- | --- | --- | --- |
| Initial | 0dg | 0dg | 0dg | 0dg | 0dg | 0dg |
| Input | VCO5 | VCO6 | VCO4 | VCO2 | VCO3 | VCO1 |
| Depth | 0.200 | 0.200 | 0.100 | 0.200 | 0.200 | 0.100 |
| Idx Inp. | CV1 | CV2 | CV3 | CV1 | CV2 | CV3 |
| Idx Max. | 0.250 | 0.250 | 0.250 | 0.250 | 0.250 | 0.250 |
| Idx Min. | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |

| ## AM | O1 | O2 | O3 | O4 | O5 | O6 |
| --- | --- | --- | --- | --- | --- | --- |
| Input | VCO3 | VCO1 | Off | VCO6 | VCO4 | Off |
| Depth | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| Bias | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| Idx Inp. | CV4 | CV5 | Off | CV4 | CV5 | Off |
| Idx Max. | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| Idx Min. | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |

| ## M1 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Input | VCO1 | VCO2 | VCO3 | Off | Off | Off |
| Level | 100 | 100 | 100 | 0 | 0 | 0 |

Clipping		Hard

Limiting		Min.

| ## M2 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Input | VCO4 | VCO5 | VCO6 | Off | Off | Off |
| Level | 100 | 100 | 100 | 0 | 0 | 0 |

Clipping		Hard

Limiting		Min.