# Dionisi preset 27 – WaveScroller

## Sequencing

Melody into V/Oct or MIDI (Channel 1) input of Dionisi.

No other input required.

Master output: Mix 1 & 2.

## Controls

“Tetrature LFO” comprised of VCO’s 4 5 6, each a triangle wave spaced 120 degrees apart respectively modulating the amplitude of VCO’s 1 2 3. VCO 1 = Sine, VCO2 = Triangle, VCO 3 = Ramp  Sine into triangle into ramp into sine,…

CV1 = LFM VCO 1 with VCO 2

CV2 = LFM VCO 2 with VCO 3

CV3 = PM VCO 3 with VCO 2

CV4 = EFM of VCO’s 4-6 (tetrature modulators)

CV5 = AM VCO’s 4-6 with VCO 1

CV6 = Detune VCO 1 2 3, each in larger amounts than the previous one

## Outputs & comments

Mix 1 = VCO 1 + VCO 2 + VCO 3  Mono VCO out

Mix 2 = VCO 4 + VCO 5 + VCO 6  Mixed modulators out

| ## Oscillator | O1 | O2 | O3 | O4 | O5 | O6 |
| --- | --- | --- | --- | --- | --- | --- |
| Range | Audio | Audio | Audio | Low | Low | Low |
| Pitch Mode | Ratio | Ratio | Ratio | Ratio | Ratio | Ratio |
| Ratio | 1.000x | 1.000x | 1.000x | 4.000x | 4.000x | 4.000x |
| Wave | Sine | Tri | Ramp | Tri | Tri | Tri |
| Quantise | Off | Off | Off | Off | Off | Off |
| Sync Input | Off | Off | Off | D1 | D1 | D1 |
| Sync State | Pos e. | Pos e. | Pos e. | Pos e. | Pos e. | Pos e. |
| Midi Chan. | 1 | 1 | 1 | 1 | 1 | 1 |
| PB Range | 0 | 0 | 0 | 0 | 0 | 0 |

| ## Detune | O1 | O2 | O3 | O4 | O5 | O6 |
| --- | --- | --- | --- | --- | --- | --- |
| Detune Quantise | Off | Off | Off | Off | Off | Off |
| Detune Amount | 0ct | 0ct | 0ct | 0ct | 0ct | 0ct |
| Fine Hz Amount | 0.0Hz | 0.0Hz | 0.0Hz | 0.0Hz | 0.0Hz | 0.0Hz |

| ## Exp FM | O1 | O2 | O3 | O4 | O5 | O6 |
| --- | --- | --- | --- | --- | --- | --- |
| Input | CV6 | CV6 | CV6 | CV4 | CV4 | CV4 |
| Depth | 0.012 | 0.008 | 0.004 | 0.500 | 0.500 | 0.500 |
| Idx Inp. | Off | Off | Off | Off | Off | Off |
| Idx Max. | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| Idx Min. | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |

| ## Lin FM | O1 | O2 | O3 | O4 | O5 | O6 |
| --- | --- | --- | --- | --- | --- | --- |
| Input | VCO2 | VCO3 | Off | Off | Off | Off |
| Depth | 0.300 | 0.300 | 1.000 | 1.000 | 1.000 | 1.000 |
| Idx Inp. | CV1 | CV2 | Off | Off | Off | Off |
| Idx Max. | 0.300 | 0.300 | 1.000 | 1.000 | 1.000 | 1.000 |
| Idx Min. | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |

| ## Phase | O1 | O2 | O3 | O4 | O5 | O6 |
| --- | --- | --- | --- | --- | --- | --- |
| Initial | 0dg | 0dg | 0dg | 0dg | 120dg | 240dg |
| Input | Off | Off | VCO2 | Off | Off | Off |
| Depth | 1.000 | 1.000 | 0.500 | 1.000 | 1.000 | 1.000 |
| Idx Inp. | Off | Off | CV3 | Off | Off | Off |
| Idx Max. | 1.000 | 1.000 | 0.500 | 1.000 | 1.000 | 1.000 |
| Idx Min. | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |

| ## AM | O1 | O2 | O3 | O4 | O5 | O6 |
| --- | --- | --- | --- | --- | --- | --- |
| Input | VCO4 | VCO5 | VCO6 | VCO1 | VCO1 | VCO1 |
| Depth | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| Bias | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| Idx Inp. | Off | Off | Off | CV5 | CV5 | CV5 |
| Idx Max. | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| Idx Min. | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |

| ## M1 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Input | VCO1 | VCO2 | VCO3 | Off | Off | Off |
| Level | 100 | 70 | 50 | 0 | 0 | 0 |

Clipping		Hard

Limiting		Min.

| ## M2 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Input | VCO4 | VCO5 | VCO6 | Off | Off | Off |
| Level | 100 | 100 | 100 | 0 | 0 | 0 |

Clipping		Hard

Limiting		Min.